Conversions and Constants

* 1 inch = 25.4 millimeters

* 1 foot = 0.3048 meters

* 1 Coulomb = 6.242x1018 electrons

* 1 Circular mil (CM) = (π/4) Square mils

* Permittivity of a vacuum = 8.84 x 10-12 F/m

* Permeability of vacuum = 1.2566370614×10-6 kg∙m/A2∙s2

* Relative Permeability of Iron = 100

* The uF unit on the schematics means micro Farad (µF)

Module 1 Equations

[1.1]	I= Qt

[1.2]	R= ρ LA

[1.3]	Rx=R200C[1+ α Tx- T200C]

[1.4]	V= I ∙ R

[1.5]	I= V ∙ G

[1.6]	IR=ΔVR=Va -Vb R

[1.7]	P=V ∙I

[1.8]	P=V2R

[1.9]	P=I2∙R

[1.10]    Psupplied=Pdissipated

[1.11]    E=P ∙t  Units: E (J), P (W), t (s)

[1.12]	RTotal=R1+ R2+ R3+ ∙∙∙RN

[1.13]	RTotal=(R1∙ R2)(R1+ R2)

[1.14]	RTotal=11R1  + 1R2  + 1R3  +∙∙∙ 1RN

[1.15]    Varound a closed loop=0

[1.16]	Vx= Vs ∙ RxRtotal

[1.17]	IIN=IOUT   (works on nodes or branches)

[1.18]	Ix= Is ∙ RtotalRx

DC Circuits Cheat Sheet

Module 3.1 and 3.2 Equations

[3.1]	C= ErEoAd

[3.2]	C= QV

[3.3]	E=12 C V2

[3.4]	iCt=C dvdt

[3.5]	vC(t)=1C t0tic(t)∙ dτ+vc(t0)

[3.6]	L= N2∙ μr ∙ μ0 ∙ Al

[3.7]	E=12 L ∙i2

[3.8]	vt=L diLdt

[3.9]	iLt=1L t0tvL∙ dτ+iLt0

Module 3.3 to 3.5 Equations

[3.10]	CTotal=C1+ C2+ C3+ ∙∙∙CN

[3.11]	CTotal=(C1∙ C2)(C1+ C2)  ONLY with 2 capacitors

[3.12]	CTotal=11C1  + 1C2  + 1C3  +∙∙∙ 1CN

[3.13]   Tau=τ=Req∙Ctotal

[3.14]   Tau=τ=LtotalReq

[3.15]   vC(t)=VS (1- e-tτ)

[3.16]   vL(t)=VS (e-tτ)

[3.17]   t=-ln(1-vC(t)VS)∙τ

[3.18]   t=-ln(vLtVS)∙τ

[3.19]   iR t=iC t=ΔvR=VS -vC(t)iNRi

[3.20]   iR t=iL t=ΔvR=VS -vL(t)iNRi