# Microwave Filters for Next Generation Radio Frequency Transceivers

## Summary

Radio frequency (RF) transceivers are one of the most important building blocks of various applications including wireless communication systems, radar, biomedical imaging and microwave transponders. Microwave/mmWave filters play a significant role in various stages of such networks such as for band selection in the receiver front-end architectures and harmonic rejection in transmitter systems. They can also be used for impedance matching networks for the RF amplifier stages to achieve wideband operations.

The filters used in the receiver front ends need to have low insertion loss values, sharp band edges, strong out-of-band rejection, and good power handling performances in order to meet the stringent requirements of the contemporary transceivers. This is mainly required to provide the minimum possible attenuation and noise figure to the system for the receiver architectures while keeping the desired signal level and suppressing the higher-order harmonics in transceiver systems. They also need sharp band edges and strong out-of-band rejection to not allow the interfering signals within the crowded frequency spectrum. For instance, a wireless receiver operating at N41 band (2.496 GHz—2.690 GHz) requires a front-end filter with a maximum of 1 dB insertion loss, 35 dBm power handling performance while having 30 dB rejection at the adjacent N40 band (2.3GHz--2.4GHz). Achieving all these tasks require the microwave resonators to have very high quality factor values, of order 1000 with strong  power handling capabilities. They also need to have small physical sizes in order to fit inside of a chip.

This study starts with introducing the significance and limitations of the frequency selective networks for microwave transceivers. This includes the inability of achieving good selectivity and insertion loss within a small physical size using electromagnetic or lumped-element based resonators. In addition, the limitations of achieving wide bandwidth and strong out-of-band rejection performances are explained in Chapter 1. Several contemporary microwave resonator technologies in Chapter 2. This includes lumped-element based on-chip resonators, electromagnetic resonators including resonant cavities, substrate integrated waveguide (SIW) technology, and active microwave resonators for on-chip filter applications. The novel methodology of estimating the quality factor of quarter-mode cavities, as published in [1] is explained. In addition, the use of substrate-integrated stepped-impedance resonators for pushing the higher-order modes away from the passband, as described in [2] and [3] is also examined. On a separate chapter (Chapter 3), microelectromechanical systems (MEMS) based acoustic-wave resonators are introduced with an emphasis on the bulk acoustic wave (BAW) resonator technology, its on-chip fabrication techniques and circuit equivalent models.

Chapter 4 examines the contemporary coupled-resonator filter design methodologies with a special emphasis on the coupling matrix method and the extracted-pole based filter design. Examples of the use of SIW based resonators for integrated microwave filters for the “horus” active electronically scanned antenna array system is described with the coupling matrix-based design methods, as published in [4]. Chapter 5 introduces the lossy coupling matrices and the lossy-active coupling matrices in the design of microwave filters for on-chip transceiver systems, as published in [5].

Proceeding with the design considerations for on-chip filters, Chapter 6 introduces the novel “hybrid acoustic-electromagnetic filters” for wideband and on-chip applications, as published in [6]. A thorough analysis of using the extracted-pole method to maximize the achievable passband with limited electromechanical coupling coefficient is made with the use of the hybrid extracted-pole method and the coupling matrices. Finally, an on-chip hybrid acoustic-electromagnetic filter for S-band applications is designed utilizing the BAW resonator of Texas Instruments in Chapter 7.

[1]: N. R. Almuqati, G. Ariturk and H. H. Sigmarsson, "Empirical Quality Factor Prediction of Quarter-Mode Substrate Integrated Waveguide Resonators," in IEEE Journal of Microwaves, vol. 1, no. 3, pp. 777-786, July 2021, doi: 10.1109/JMW.2021.3084859.

[2]: S. Saeedi, G. Ariturk and H. H. Sigmarsson, "Compact Wide-Stopband Bandpass Filter Based on Highly-Loaded Substrate Integrated Cavity Resonators," 2020 IEEE Radio and Wireless Symposium (RWS), San Antonio, TX, USA, 2020, pp. 145-147, doi: 10.1109/RWS45077.2020.9050048.

[3]: G. Ariturk, N. R. Almuqati, S. Saeedi and H. H. Sigmarsson, "Board-Integrated Tunable Bandpass Filter with Wide Stopband for Phased Array Antennas," 2021 IEEE 21st Annual Wireless and Microwave Technology Conference (WAMICON), Sand Key, FL, USA, 2021, pp. 1-4, doi: 10.1109/WAMICON47156.2021.9443585.

[4]: G. Ariturk, N. R. Almuqati and H. H. Sigmarsson, "Element-Level Microwave Filter Integration in Fully-Digital Phased Array Radar Systems," 2022 IEEE 22nd Annual Wireless and Microwave Technology Conference (WAMICON), Clearwater, FL, USA, 2022, pp. 1-4, doi: 10.1109/WAMICON53991.2022.9786104.

[5]: G. Ariturk and H. H. Sigmarsson, "Lossy Microwave Filters With Active Shape Correction," in IEEE Access, vol. 9, pp. 35075-35087, 2021, doi: 10.1109/ACCESS.2021.3061999.

[6]: G. Ariturk, N. R. Almuqati, Y. Yu, E. T. -T. Yen, A. Fruehling and H. H. Sigmarsson, "Wideband Hybrid Acoustic-Electromagnetic Filters with Prescribed Chebyshev Functions," 2022 IEEE/MTT-S International Microwave Symposium - IMS 2022, Denver, CO, USA, 2022, pp. 887-890, doi: 10.1109/IMS37962.2022.9865475.